BRILLIANT METAL WORKS

Custom Metal Design & Fabrication

Project Management &

Estimation System

USER GUIDE

Live Estimation       Section Optimizer       Margin Control       Rhino/GH Export

Version 1.0   •   May 2026   •   Kigali, Rwanda

# 1.  System Overview

This system is your central tool for managing every railing and metalwork project — from the first client enquiry through to final installation. It removes guesswork from estimating, protects your margins, and produces professional client quotations in seconds.

## What the system does

• Calculates material quantities — steel profiles, infill sheets, catch frames — automatically from dimensions

• Optimises panel sections — finds the cheapest combination of sheet size and number of sections

• Controls your pricing — adds consumables, surface treatment, transport and a configurable margin

• Generates client quotations — clean PDF with high-level categories — no internal cost breakdown shown

• Exports to Rhino/Grasshopper — one click sends exact geometry data to your CAD model

## The three-tab workflow

| STEP 1 Site Survey | STEP 2 Estimation | STEP 3 Quotation |
| --- | --- | --- |
| Enter dimensions, select profiles and infill type. The system calculates everything live as you type. | Review the detailed material and labour breakdown. Add variable costs and set your margin. See the full pricing chain. | Preview the client document, download the PDF. The client sees professional categories — not your internal costs. |

## Project status flow

Every project moves through eight stages. Each stage is one click — you control when to advance.

| Inquiry | Site Survey | Design | Quote Sent | Approved | Fabrication | Installation | Completed |
| --- | --- | --- | --- | --- | --- | --- | --- |

ℹ  You can also jump directly to any status by clicking it in the stepper bar at the top of a project page.

# 2.  Dashboard & Project Management

## Creating a new project

From the dashboard, click + New Project. Fill in:

• Project code — assigned automatically (e.g. BAL-001 for balconies, STA-001 for staircases)

• Client name and phone number

• Location of the site

• Project type — Balcony or Staircase

## Dashboard — search and filter

The dashboard shows all projects in a single table. Two tools help you find what you need quickly:

• Search bar — type any part of the project code, client name, title or location

• Status filters — click any status badge to show only projects at that stage

The project count updates live as you filter.

## Project types

| Type | What it covers |
| --- | --- |
| Balcony | Horizontal railings — any length, wall or floor mounted, one or multiple sections |
| Staircase | Inclined railings along a stringer — rise, run, one or both sides, parallelogram panels |

# 3.  Site Survey — Balcony

## Dimensions

| Field | Unit | What to measure |
| --- | --- | --- |
| Total Length | mm | Full run of the railing from end post to end post |
| Total Height | mm | Top of rail to floor/slab — typically 900–1100mm residential |

## Section Layout Optimizer

This is the most important feature of the survey tab. Instead of guessing how many sections to use, the system calculates every possible option (from 2 sections up to ~20) and ranks them by total material cost.

| Sections | Section W | Cut Size | Sheet | Sheets | Total Cost |
| --- | --- | --- | --- | --- | --- |
| 3 | 1940mm | 1960×1020mm | 2000×1000 | 3 | RWF 42,000 |
| 4 | 1440mm | 1460×1020mm | 2000×1000 ×2 | 2 | RWF 38,500 |
| 5 | 1140mm | 1160×1020mm | 2440×1220 | 5 | RWF 51,000 |

Click any row to select that layout. The row turns black to confirm your choice. The BEST badge marks the cheapest option. A blue "×N" means multiple panels nest on one sheet — those options are often the most economical.

ℹ  Changing any dimension, profile or infill type automatically recalculates all options. You must re-select a layout after any change.

## Steel Profiles

| Profile | Size | Typical use |
| --- | --- | --- |
| 20×20 SHS | 20 × 20mm | Inner catch frame (standard) |
| 40×20 RHS | 40 × 20mm | Bottom rail — slim horizontal |
| 40×40 SHS | 40 × 40mm | Posts and top rail — most common |
| 60×40 RHS | 60 × 40mm | Heavy-duty posts or top rail |

## Infill Types

• Laser-Cut Sheet — plain steel sheet — section optimizer active, catch frame required

• Glass Panels — toughened glass — manual post spacing, no catch frame

• Flat Bars — horizontal bar infill — manual post spacing, bar spacing input

## Mounting & Site

• Wall-mounted — specify wall type (concrete, brick, block) — affects hardware cost recommendations

• Floor-mounted — base plate welded to floor slab

• Access Difficulty — Easy / Medium / Difficult — inform this from your site visit, affects labour estimate

# 4.  Site Survey — Staircase

## Staircase dimensions

For a staircase railing, you are not building the stairs — you are building the railing along them. Measure the structural envelope of the stair flight.

| Field | Unit | What to measure |
| --- | --- | --- |
| Total Rise | mm | Vertical height from ground to top landing — e.g. 2800mm |
| Total Run | mm | Horizontal distance from first step to last — e.g. 3500mm |
| Width | mm | Usable stair width — e.g. 1200mm |
| Handrail Height | mm | Height of top rail above stringer line — typically 1000mm |
| Number of Flights |  | 1 = straight stair, 2 = with intermediate landing |
| Rail Sides |  | One side only, or both sides of the stair |

ℹ  The system calculates stringer length automatically: √(rise² + run²). You do not need to measure along the slope.

## Section optimizer — inclined panels

Staircase panels are parallelograms, not rectangles — they follow the slope of the stringer. The optimizer works along the stringer length and calculates:

• Section slope spacing — distance between posts measured along the slope

• Horizontal span — the structural span between posts — flagged if it exceeds 2000mm

• Cut slope edge × cut height — exact parallelogram dimensions for the laser/CNC cutter

• Bounding rectangle — the sheet area needed to cut one panel — used for sheet nesting

ℹ  The bounding height is larger than the panel height because of the slope offset. The optimizer accounts for this automatically.

## CNC Panel dimensions

After choosing a section layout, the Estimation tab shows a blue card with the exact cutting dimensions for all panels. All panels in one flight are identical — rotate alternate panels 180° when nesting to save material.

# 5.  Estimation & Pricing

## Material & Labour Breakdown

The top table on the Estimation tab shows every cost line with quantity, unit and unit price — this is your internal view. The client never sees this table.

| Line item | What it covers |
| --- | --- |
| Steel Frame | Posts, top rail, bottom rail, catch frame — priced per kg |
| Infill Sheets / Glass / Bars | Panel material — priced per sheet or per m² |
| CNC Cutting | Laser or plasma cut — priced per metre of cut length |
| Labour | Fabrication and welding days — priced per day |

## Variable Costs

Below the breakdown, the Variable Costs card covers the hidden costs that often cause underquoting:

| Cost | Default | How it works |
| --- | --- | --- |
| Design + Laser Cutting | Manual | Enter the actual cost from your cutting supplier |
| Installation | Manual | Labour cost for on-site fitting — varies by job |
| Welding Consumables | 7% of frame | Auto-calculated — adjust the % slider if your usage differs |
| Surface Treatment | None | Select Powder Coat or Paint — enter rate per m²; cost auto-calculates from infill area |
| Transport & Delivery | Manual | Flat fee — enters your fuel and vehicle cost |
| Hardware & Fixings | Manual | Anchors, bolts, rawl plugs — flat fee per job |

## The Pricing Chain

This is the core of the margin system. Every number flows in one direction — there are no hidden overrides.

Materials + Labour subtotal (from the breakdown table) + Welding consumables + Surface treatment + Transport + Hardware + Design + Installation = Your direct cost + Contingency (default 10%) covers material price risk and rework = Adjusted cost + Business margin (default 30%) overhead, equipment, profit = QUOTED PRICE ← this is what the client pays

ℹ  Both the contingency % and business margin % are editable inputs — you can adjust them per job. The defaults (10% + 30%) are set to protect you from the most common causes of underquoting.

## Why 30% margin is not "too high"

A 30% business margin on top of direct + contingency costs covers:

• Workshop rent, electricity, equipment maintenance

• Owner and management time not billed as direct labour

• Guarantee and warranty call-backs

• Slow periods when direct costs still run

If your margin feels high, check whether these costs are already included in your labour rate. If they are not, 30% is the minimum safe level.

# 6.  Quotation Generation

## What the client sees vs what you see

The system keeps two views completely separate. Your internal cost data never appears in the client document.

| Your internal Estimation tab | Client PDF & preview |
| --- | --- |
| • Each item with qty, unit, unit price • Frame cost, infill cost, labour cost • Consumables % and computed amount • Surface treatment rate per m² • Full pricing chain (direct → margin) | • Steel Frame & Fabrication • Infill Panels • Surface Treatment (if used) • Transport & Delivery (if used) • Installation (if used) Each line shows only the proportional quoted amount — no rates. |

## Quotation settings

• Timeline (weeks) — how long from deposit to delivery — appears on the PDF

• Payment Terms — default: 50% deposit on approval, 50% on completion — editable

• Scope Notes — any additional notes you want the client to see

## Downloading the PDF

Click Download PDF in the Quotation tab. The PDF is generated instantly and named after the project code — e.g. BAL-001_quotation.pdf.

The PDF is ready to send directly to the client. It shows your company name, project reference, valid-until date, all line items with amounts, payment terms and scope.

ℹ  Save the quotation first (Save Quotation button) before downloading if you want the settings stored in the database.

## Proportional amounts — how they work

Each client line item amount is calculated as: (raw cost ÷ your direct total) × quoted price. This means the total of all client line items always equals the quoted price exactly. Rounding is absorbed in the last line item.

# 7.  Rhino / Grasshopper Integration

## Exporting the geometry data

Once the site survey is complete, click Export to Rhino (.json) in the sidebar or header. A JSON file is downloaded named after the project code.

This file contains all the dimensional data the Grasshopper scripts need — nothing has to be retyped in Rhino.

## JSON file contents

| Section | What it contains |
| --- | --- |
| geometry | Total dimensions, stringer length, slope angle, step count and rise/going |
| handrail | Post count, number of panels, height, rail sides, catch profile |
| profiles | Post, top rail, bottom rail profile strings (e.g. "40x40") |
| infill | Infill type, sheet thickness, sheet size chosen by optimizer |
| panel_cnc | Cut slope edge, height, bounding rectangle, interior angle, total panels |

## Grasshopper scripts — two components

There are two Python scripts for each project type (balcony and staircase). Paste each into a separate GHPython component in Grasshopper, then wire them together.

| Script | What it does |
| --- | --- |
| balcony_reader.py / staircase_reader.py | Reads the JSON file. Connect the json_path input to a Panel containing the file path. Outputs all parameters as individual wires. |
| balcony_geometry.py / staircase_geometry.py | Receives parameters from the reader. Outputs geometry objects: stringer line, step treads, posts, rails, infill surfaces, catch frame lines, CNC flat panels. |

## Grasshopper outputs — what you see in Rhino

| Output | Description |
| --- | --- |
| stringer_line | The main diagonal line from floor to top landing |
| step_lines | All tread and riser lines for visual reference |
| post_lines | Vertical handrail post lines at each section boundary |
| top_rail_line / bottom_rail_line | Polylines along the top and bottom of the handrail |
| infill_surfaces | The inclined panel surfaces between posts (3D, in-place) |
| catch_frame_lines | The 4 inner catch frame bars per panel — inset from posts and rails |
| cnc_panels | Flat parallelogram surfaces laid out for CNC — all identical |
| cnc_bounding_rects | Bounding rectangles showing the sheet cut area per panel |
| report | Text output with full material schedule, cut dimensions and sheet fit check |

ℹ  Use the reload toggle (Button component) in Grasshopper to force a re-read of the JSON file whenever you update the survey data in the system and export a new file.

## Workflow in practice

1.  Complete the site survey in the system.

2.  Click Export to Rhino — save the JSON into your project folder.

3.  Open Grasshopper — connect the reader script to the JSON path.

4.  Wire reader outputs to the geometry script inputs.

5.  The full 3D railing appears in Rhino. Adjust the origin point to place it correctly in your model.

6.  Use the report text output to confirm all dimensions match your quotation.

7.  Send cnc_panels to the laser cutter nesting software.

## Quick reference — default values

| Setting | Default | When to change |
| --- | --- | --- |
| Welding consumables | 7% | Increase for complex weld patterns or premium wire |
| Powder coat rate | 8,000 RWF/m² | Get a current quote from your coating supplier |
| Paint rate | 5,000 RWF/m² | Adjust for primer + topcoat combinations |
| Contingency | 10% | Raise to 15% for complex or uncertain jobs |
| Business margin | 30% | Review quarterly against your actual overhead costs |
| Quotation validity | 30 days | Shorten if steel prices are volatile |
| Payment terms | 50% / 50% | Standard for Rwanda market — adjust per client |

Brilliant Metal Works — Project Management & Estimation System   •   User Guide v1.0   •   May 2026